Product / Process Change Notice

Parts Affected:

Chip process CP195, NPN small signal transistors, wafers, and die in chip form.

Extent of Change:

The CP195 wafer process has been discontinued and replaced with the CP318V wafer process. The CP195 chip process measures 30 x 30 mils and the CP318V chip process measures 26 x 26 mils. See Figures 1 and 2 for die geometry.

Reason for Change:

An alternate wafer foundry was approved for NPN small signal transistor processes.

Effect of Change:

This change does not affect the electrical characteristics of any device.

- Qualification:

Standard evaluation and qualifications completed resulting in no electrical rejects.

- Effective Date of Change:

Existing inventory will be shipped until depleted.

- Sample Availability:

- Please contact Salesperson or Manufacturer’s Representative.

Figures:

Figure 1: CP195 Chip Geometry				        Figure 2:CP318V Chip Geometry

Die Size:		30 x 30 mils

Die Thickness:		7.5 mils

Bond Pad Area (Emitter):	8 x 8 mils

Bond Pad Area (Base):	7.2 x 7.2 mils

Topside Metal:		Al (30,000Å)

Backside Metal:		Au (11,000Å)

Die Size:		26 x 26 mils

Die Thickness:		7.1 mils

Bond Pad Area (Emitter):	5.5 x 5.5 mils

Bond Pad Area (Base):	5.5 x 5.5 mils

Topside Metal:		Al-Si (17,000Å)

Backside Metal:		Au (12,000Å)

Part Numbers Affected:

CP195-2N3501-CT

CP195-2N3501-WN

CP195-2N5682-CT

CP195-2N5682-WN

2N3501

2N5682