Product / Process Change Notice

- Parts Affected:

Chip process CP517, PNP silicon, Darlington power transistors, wafers, and bare die.

- Extent of Change:

The CP517 wafer process has been discontinued and replaced with the CP527 wafer process.

The overall wafer diameter is being reduced from 5 inch to 4 inch.

The die pattern has been changed; see figures 1 and 2 for details.

- Reason for Change:

This process was transferred to an alternate wafer foundry due to low volume production.

- Effect of Change:

The wafer process meets all electrical specifications of the individual devices listed on the following page.

- Qualification:

Standard evaluation and qualifications completed resulting in no performance degradation.

- Effective Date of Change:

Existing inventory will be shipped until depleted.

- Sample Availability:

- Please contact Salesperson or Manufacturer’s Representative.

- Figures:

Figure 1: CP517 Chip Geometry (Discontinued)			         Figure 2:CP527 Chip Geometry

Wafer Diameter:		5 inch

Die Size:		111 x 111 mils

Die Thickness:		11 mils

Bond Pad Size (Emitter):	20 x 26 mils

Bond Pad Size (Base):	20 x 30 mils

Topside Metal:		Al (60,000Å)

Backside Metal:		Au/Cr/Ni/Au (6,100Å)

Wafer Diameter:		4 inch

Die Size:		110 x 110 mils

Die Thickness:		10.6 mils

Bond Pad Size (Emitter):	24 x 42 mils

Bond Pad Size (Base):	21 x 24 mils

Topside Metal:		Al (20,000Å)

Backside Metal:		Ag (10,000Å)

Part Numbers Affected: