Product / Process Change Notice

- Parts Affected:

Chip process CP707, PNP silicon small signal Darlington transistors, wafers, and bare die.

- Extent of Change:

The CP707 wafer process has been discontinued and replaced with the CP727V wafer process.

The overall wafer diameter was increased from 4 inch to 5 inch.

The overall wafer thickness was reduced from 9 mils to 7.1 mils.

The backside metallization (Au) was reduced from 18,000Å to 9,000Å.

The die size was reduced from 27 x 27 mils to 22.8 x 22.8 mils.

The die pattern and bond pad area were changed; see figures 1 and 2 for details.

- Reason for Change:

Process transfer from the 4” wafer fab to the 5” wafer fab.

- Effect of Change:

This change does not affect the electrical characteristics of any device.

- Qualification:

Standard evaluation and qualifications completed resulting in no performance differences compared to current product.

- Effective Date of Change:

May 1, 2015. Existing inventory will be shipped until depleted.

- Sample Availability:

- Please contact Salesperson or Manufacturer’s Representative.

- Figures:

Figure 1: CP707 Chip Geometry (Discontinued)		   Figure 2: CP727V Chip Geometry

Wafer Diameter:		4 inch

Die Size:		27 x 27 mils

Die Thickness:		9.0 mils

Bond Pad Size (Emitter):	5.3 x 6.5 mils

Bond Pad Size (Base):	5.3 x 3.8 mils

Topside Metal:		Al (30,000Å)

Backside Metal:		Au (18,000Å)

Wafer Diameter:		5 inch

Die Size:		22.8 x 22.8 mils

Die Thickness:		7.1 mils

Bond Pad Size (Emitter):	4.7 x 4.7 mils

Bond Pad Size (Base):	4.7 x 4.7 mils

Topside Metal:		Al-Si (17,000Å)

Backside Metal:		Au (9,000Å)

Part Numbers Affected: