Product / Process Change Notice

- Parts Affected:

Chip process CP547, PNP silicon, Darlington power transistor wafers, and bare die.

- Extent of Change:

The CP547 wafer process has been discontinued and replaced with the CP647 wafer process.

The overall wafer diameter is being reduced from 5 inch to 4 inch.

The die size and pattern has been changed; see figures 1 and 2 for details.

- Reason for Change:

This process was transferred to an alternate wafer foundry due to low volume production.

- Effect of Change:

The wafer process meets all electrical specifications of the individual devices listed on the following page.

- Qualification:

Standard evaluation and qualifications completed resulting in no performance degradation.

- Effective Date of Change:

May 4, 2015 - Existing inventory will be shipped until depleted.

- Sample Availability:

- Please contact Salesperson or Manufacturer’s Representative.

- Figures:

Figure 1: CP547 Chip Geometry (Discontinued)			    Figure 2:CP647 Chip Geometry

Wafer Diameter:		5 inch

Die Size:		195 x 195 mils

Die Thickness:		11.4 mils

Bond Pad Size (Emitter):	61 x 35 mils

Bond Pad Size (Base):	29 x 29 mils

Topside Metal:		Al (60,000Å)

Backside Metal:		Ti/Ni/Au (1,100Å/4,000Å/500Å)

Wafer Diameter:		4 inch

Die Size:		211 x 211 mils

Die Thickness:		12.5 mils

Bond Pad Size (Emitter):	60 x 50 mils

Bond Pad Size (Base):	49 x 41 mils

Topside Metal:		Al (50,000Å)

Backside Metal:		Ti/Ni/Ag (1,000Å/6,000Å/10,000Å)

Part Numbers Affected