Product / Process Change Notice

- Parts Affected:

3KW Transient Voltage Suppressors in the SMC case.

- Extent of Change:

Thermal oxide deposition was deployed underneath the original oxide in order to improve the interface. See Figure 1.

- Reason for Change:

Silicon fractures were observed on the die after IR reflow which resulted in leakage failures.

- Effect of Change:

Oxide was strengthened by utilizing a high temp oxidation process; a thermal oxide deposition was deployed underneath the original oxide.

- Qualification:

1000 devices each of 3SMC13CA and 3SMC18CA were subjected to 6 cycles of IR reflow. All devices passed DC Electrical characteristics including Clamping Voltage. No silicon fractures were observed on the die surface.

- Effective Date of Change:

- January 12, 2015

- Sample Availability:

- Please contact Salesperson or Manufacturer’s Representative

Figure 1:

Original

New

Part Numbers Affected:

| 3SMC5.0A | 3SMC5.0CA | 3SMC33A | 3SMC33CA |
| --- | --- | --- | --- |
| 3SMC6.0A | 3SMC6.0CA | 3SMC36A | 3SMC36CA |
| 3SMC6.5A | 3SMC6.5CA | 3SMC40A | 3SMC40CA |
| 3SMC7.0A | 3SMC7.0CA | 3SMC43A | 3SMC43CA |
| 3SMC7.5A | 3SMC7.5CA | 3SMC45A | 3SMC45CA |
| 3SMC8.0A | 3SMC8.0CA | 3SMC48A | 3SMC48CA |
| 3SMC8.5A | 3SMC8.5CA | 3SMC51A | 3SMC51CA |
| 3SMC9.0A | 3SMC9.0CA | 3SMC54A | 3SMC54CA |
| 3SMC10A | 3SMC10CA | 3SMC58A | 3SMC58CA |
| 3SMC11A | 3SMC11CA | 3SMC60A | 3SMC60CA |
| 3SMC12A | 3SMC12CA | 3SMC64A | 3SMC64CA |
| 3SMC13A | 3SMC13CA | 3SMC70A | 3SMC70CA |
| 3SMC14A | 3SMC14CA | 3SMC75A | 3SMC75CA |
| 3SMC15A | 3SMC15CA | 3SMC78A | 3SMC78CA |
| 3SMC16A | 3SMC16CA | 3SMC85A | 3SMC85CA |
| 3SMC17A | 3SMC17CA | 3SMC90A | 3SMC90CA |
| 3SMC18A | 3SMC18CA | 3SMC100A | 3SMC100CA |
| 3SMC20A | 3SMC20CA | 3SMC110A | 3SMC110CA |
| 3SMC22A | 3SMC22CA | 3SMC120A | 3SMC120CA |
| 3SMC24A | 3SMC24CA | 3SMC130A | 3SMC130CA |
| 3SMC26A | 3SMC26CA | 3SMC150A | 3SMC150CA |
| 3SMC28A | 3SMC28CA | 3SMC160A | 3SMC160CA |
| 3SMC30A | 3SMC30CA | 3SMC170A | 3SMC170CA |

- As per JEDEC standard JESD46, Customer Notification of Product/Process Changes by Solid-State Suppliers, a lack of acknowledgement of a PCN within thirty (30) days constitutes acceptance of the change.

- The undersigned acknowledges and accepts Central Semiconductor’s Product/Process Change Notification (PCN).

| - Company Name: |  |
| --- | --- |
| - Address: |  |
| - Printed Name: |  |
| - Title: |  |
| - Signature: |  |
| - Date: |  |